JAKE'S CHAIR ASSEMBLY INSTRUCTIONS

Assuming no knots and 14' or longer lengths, the

following amounts of wood adequate to build one Jake's Chair-

Lumber list- see discussion below

Jake's Chair only

(32) linear feet of 12" wood, OR

(40) linear feet of 10" wood, OR

(36) linear feet of  8" wood + (6) linear feet of 10" wood

A 27" wide footstool requires the following:

(10) linear feet of 10" wood, OR

(12) linear feet of 8" wood

Hardware list- galvanized deck-type or stainless steel screws

(read entire article before purchasing)

LENGTH            JAKE'S CHAIR    Footstool     Total

1-1/4"               16            8         =   24

1-5/8"               68           36         =  108

2"                    4            2         =    6

3-1/2"                4

4" pan head (10-24 or 1/4-20) bolts with nuts and washers= 4

(10) 3/4" nail-on nylon button-type glides

Before ordering or purchasing any screws, however, please take the

time to read about screw selection in the body of the article.  It is

quite important.

Other material

outdoor carpenters glue- pint (titebond II, Gorilla Glue or

even Liquid Nails for Decks)

4 sheets 80 grit sandpaper

1/2 sheet 1/4" luan plywood or hardboard for pattern

Tools (minimum for practical use)

electric drill/screwdriver

1/8" bit

#6 countersink

Phillips or square screwdriver bit (to match screws)

circular saw

saber saw

sander

router with piloted 3/8" rounding over bit

Material

JAKE'S CHAIR can be built out of virtually any wood.  If your

plans include painting JAKE'S CHAIR, pine (SPF) lumber can be

used.  For a natural look outdoors, there are several woods

that work well.  I have used Cypress for the chairs on my own

deck, but have seen rough sawn Western Red Cedar used as

well.  Hardwoods, such as Oak, Mahogany or Pau Lope are also

good choices, but difficult to always locate in adequate widths.

Where available, Redwood or Juniper would also work well.

Pine- If the chair is to be painted, the most economical

choice is #2 SPF (Spruce, Fir, Pine) in 1" X 10" or 1" X 12"

widths.  The resulting finished lumber will thus measure 3/4"

X 9-1/2" or 11-1/2".  Lately, there has been a premium on the

12" width and I advise against wasting the money.  I prefer

to pick through the lumber at the lumber yard and buy the

boards in 14' or longer lengths.  Since the lumber is so

inexpensive, you can afford to "cut around" knots. Be sure

that the boards aren't very cupped, since they cannot be used

for the larger pieces.  If some of the SPF is cupped, use it

for the narrower parts, and not the legs, seat frame or arms.

Redwood- This soft outdoor wood works well for JAKE'S CHAIR.

Redwood is usually available in the preferred wider widths

and comes S4S (surfaced four sides).  While costing usually

more per board foot, there is some economy in premium

Redwood, since there are no knots to "cut around."

The S4S finish is also quite comfortable against the skin

when using JAKE'S CHAIR.

Western Red Cedar- This inexpensive wood is almost always

sold in its rough sawn form.  That means that you will either

have to learn to love the feel of the rough wood on your arms

and back, or will have to sand or plane it smooth.  A friend

of mine built several JAKE'S CHAIRS from the rough sawn

material, and while they looked nice, I did not like the

feel.  In my own shop, I have a planer that will quickly

smooth the wood.  However, since 1" Western Red Cedar is

usually sold rough sawn 3/4" thick or even less, the planed

material is almost too thin to be used in the legs or seat

frame.  Overall, I discourage its use, unless you can obtain

it in S4S finish in 4/4 thickness.

Cypress- This wood is easily obtained along the East Coast,

but becomes more "exotic" inland.  It is as easy to work with

as Western Red Cedar, but lasts practically forever outdoors.

I prefer to purchase it in 4/4 thickness (this is a lumber

term meaning that is a full 1" thickness and is pronounced

"four quarter") and plane it slightly to smooth it up.  In

North Carolina, 4/4 Cypress in random widths costs about $2

per board foot.  I have purchased it as cheaply as $1.35/bf

and for as much as $2.35/bf.  (A board foot of lumber is one

square foot of material 1" thick) You have to hunt for

Cypress wider than 8", but it is usually available.  I

usually save the wider material for the arms and seat frame,

and use the narrower material for the balance of the chair.

Juniper- This is also available at lumber yards offering

alternative woods to CCA treated lumber for outdoor use.  It

is a tiny bit softer than Cypress, but planes up beautifully

with a more pronounced grain structure.  It is slightly

redder in its freshly sanded color, but ages like cypress.

Oak- If you have a planer, rough Oak is a reasonable material

to use for JAKE'S CHAIR.  However, if you can only locate

white or red oak in its S4S finished form at a hardware store

or lumber yard, it's just not worth the price.  Unfinished

4/4 white oak costs about $2.50 per board foot around here,

while the S4S material (usually gift wrapped) costs as much

as four times as much.  Unlike the above woods, the oak

should be painted or varnished with spar varnish to protect

it from moisture.

Pau Lope- I have just learned about this exotic outdoor wood

and am anxious to try it.  It is almost four times as strong

as pine, won't burn, has the weathering ability of concrete

and the look of mahogany.  On the downside, it appears to be

quite hard and care has to be taken in predrilling every

hole.  Carbide tools also appear to be required.

Screws- The only other materials required are the four basic

screw lengths used in JAKE'S CHAIR.  These are #6 or #8

galvanized wood screws.  The lengths are 1-1/4", 1-5/8"

2" and 3-1/2".  The shortest screws are used to join material

side-to- side, while the 1-5/8" ones are used to fasten the arms,

braces, seat slats and back in place. The slightly greater

length gives increased holding power in the softer woods.

The longest screws are used to fasten and reinforce the upper

back support to the Arm Runners.  The finest screws are

the galvanized or stainless ones from McFeeley's in Lynchburg,

VA.  They come with the preferred square or "Robertson"

drive heads.

On the footstool, the 1-1/4" screws are again used to join

the ends to the riser, while the 1-5/8" screws join the slats

to the ends and hold the support in place.  The one exception

is the 2" screw inserted through the end and riser into the

upper part of the support, and used in attaching the lower

back support to the seat frame.

If you use Western Red Cedar, Redwood or Mahogany consider only using stainless steel screws, since galvanized screws will

blacken the wood near the screw.

I personally recommend drilling clearance holes and countersinks

in all selections of wood and particularly where the screw is close

to the end of the pieces, as in the seat slats and braces.

SCREWS- #8 RECOMMENDED

LENGTH       JAKE'S CHAIR    Footstool     Total

1-1/4"            16           8            24

1-5/8"            68          36           108

2"                 4           2             6

3-1/2"             4                         4

4"                 4 pan head 10/24 or 1/4-20 bolt

Stainless steel sheet metal-type flush wood screws are also

available in similar lengths.  However, where the galvanized

deck screws cost less than $.03 each, the stainless steel

screws are about $.15 each.  If you choose brass, which I

discourage, the cost jumps to about $.25 each. Galvanized

screws will eventually darken the wood where they contact it,

while the more expensive stainless steel screws will not.

You have to decide if the additional cost and time drilling

countersink holes is worth it. Remember, western red cedar

or Mahogany REQUIRE stainless steel screws to prevent

blackening around the hole.

Tool list- in order of importance

3/8" variable speed reversible drill

#2 Phillips screwdriver bit for drill

1/8" bit for pilot holes for deck screws

#6 combination countersink and pilot for wood screws- not

recommended

7-1/4" circular saw

combination blade- carbide preferred

rip fence for saw

Jig saw appx. 14 TPI blade

Router- 1hp or more recommended with 1/4" collet

3/8" carbide bearing piloted rounding over bit with 1/4"

shaft

Router pad- carpet padding will do

Belt sander- 3" by 21" belt sander is very handy to smooth

edges and surfaces before assembly.  However, a sanding

block will work also.

Palm finishing sander- very useful to finish sand the

surfaces

Step ONE               Plan  Layout

Print each of the .bmp drawings of the parts.

The method I prefer is to first enlarge the drawings using standard

drawing tools and drafting techniques.  There are no difficult angles,

curves or critical shapes.  Take your time, have fun and don't worry

if an ogee isn't precisely like shown on the drawing.  You can do the

drawing of the parts directly onto 3/16" luan mahogany plywood

Another technique would be to draw the parts full-size on paper,

then glue the full-sized plan to a piece of thin material.  While

scaling-up the plans on a computer and

printing them out full-size is possible, the simplicity of each

part of Jake's Chair and footstool make it easier and faster to

simply redraw the pieces full size.

Use either 3/16" tempered hardboard or 1/4" luan

plywood as pattern stock.  Most lumber yards have the

thinner materials available in 4' by 4' sheets as well as the

larger 4' by 8' sizes.

The shapes are then cut out with the jigsaw and the edges sanded

smooth.

If you're typical, you'll be building many Jake's Chairs, so

take your time and be careful in scaling the plans up and

making your templates out of hardboard or plywood.  It is

practically guaranteed that neighbors and friends will love

the chair and want you to make others for them, so hang onto

your templates.  Jake's Chair and footstool are very

"forgiving" of inaccuracies and mistakes.  If you make a

small mistake in scaling the plans up, you still will

probably end up with a great chair.

Step TWO             Make sanding board

Even with the 4' by 4' thin pattern material, there will be a

scrap left.  That scrap is valuable, since most folks do not

have a long-bed planer or table sander.  By gluing four

sheets of 80 grit sandpaper end to end on the scrap, the

resulting 8-1/2" by 44" sandpaper covered board is ideal for

truing edges to eliminate any waves.  Lay the sanding board

on the workbench or floor and rub the cutoff edge of the wood

or pattern back and forth over the sanding board to

straighten any edges.

Step THREE           Copying Patterns to Lumber

Note: be sure to read "Layout of Back "

before copying the pattern for the back to lumber.

Since the widths of the material can vary, it is impractical

to provide a layout of the pattern onto lumber.  In addition,

if knots are encountered, any scheme is thrown off.  However,

some pieces can be nested to eliminate waste.  The seat

frames and arms are best cut from 10" material, where they

can nest well. Likewise, while the back supports can be

nested on 10" material, placing them individually on narrower

material with other odd shapes, such as that angle braces on

the "inside" of the curve works better for me.  Since the

wider wood is generally more expensive per board foot than

the narrower cuts, I prefer to buy the minimum amount of 10"

material, and concentrate on the more available 8" widths for

the majority of the pieces.

Assuming no knots and 14' or longer lengths, I have found the

following amounts of wood adequate to build one Jake's Chair-

(32) linear feet of 12" wood, OR

(40) linear feet of 10" wood, OR

(36) linear feet of  8" wood + (6) linear feet of 10" wood

A 27" wide footstool requires the following:

(10) linear feet of 10" wood, OR

(12) linear feet of 8" wood

The footstool may also be built less than 27" wide (the width

of the chair).  I have built them as narrow as 20" with good

looking results.  Reducing the width of the footstool to 20"

saves 2' of length for either of the above widths. <<view

deck1.bmp, which shows both 27" and 20" footstools in

foreground>>

I have tried to eliminate the 10" requirement for cutting out

the seat frames (8" material is 7-1/2" wide), but reducing

the depth of the seat frame beyond what I show makes them too

weak.  However, if someone is brave enough to try a hardwood,

such as Pau Lope or Oak, they might reduce the depth of the

seat frame by that additional 1/2".  If you simply cannot

locate any 10" lumber, lay the bottom of the seat frame along

the edge of the 8" material and reduce the height of the stop

on the top of the seat frame.  <<view side1.bmp to see the

stop>>  SOME of the stop is necessary to orient the lower

back support, but since most of it is not visible from the

front of Jake's Chair, having it not match the height of the

lower back support is not too terrible.  Sanding the mismatch

in height further reduces the problem.

The only adjustments to the plans that should be made, but

are not absolutely necessary, depend on the finished

thickness of the lower back support and arm runners.

The two "ears, or tips" of the lower back support rest on the

seat frame against the stops, << view side1.bmp to see ears

and stops >> and should be the same width as the frame.

Likewise, the thickness of the arm runners determine the

overall length of the upper back support. << view back1.bmp

to see relationship>> As shown on the plans, it is 25-1/2"

long, since two 3/4" thick runners were assumed for an

overall 27" width.  If thicker or thinner material is used,

subtract the width of TWO arm runners from 27" to determine

the proper length of the upper back support. If you're off by

a bit, nobody will notice.

***WARNING- Remember that the saw blade removes about 1/8" of

kerf per cut.  As a result, straight pieces that will be

ripped MUST be separated by 1/8" when laying the pattern out.

Likewise, the ends of the material will be reduced by the

same 1/8" kerf, so leave adequate room between the pieces on

the board.

After building your first Jake's Chair and footstool, you

will find that it isn't always necessary to lay out all the

rectangular pieces, such as seat slats, legs, back etc.

Instead, an experienced builder will simply set the rip guide

on his circular saw to the desired width, and will rip the

appropriate length of material.  Once ripped, the material is

then cut to length.

Layout of Back

The back of Jake's Chair consists of seven boards.  The

boards are cut square on the bottom, but the tops are cut so

that when assembled, the boards form a rounded shape.

While the pattern shows the final rounded shape and rough

shape, it is easier, and more accurate, in my opinion, to

directly copy the curve to the boards.  Here's how:

Rather than cut the seven back pieces to the length and shape

shown, the preferred way is to cut seven boards for the back

to a width of 3-1/4" and a uniform length of 31" long.  This

is far easier for layout.

Then, place the boards together, face down,  on a flat

surface.  The corners of the bottom of the back slats touch.

The top pieces are flared to a width of 28" across the top

BEFORE MARKING THE RADIUS.  Be careful that

when the seven slats are laid out, that the gap between

the tops of all seven boards is uniform and that the center

of the center slat is 14" from the top outside corners.

Since the top pieces are flared, and the bottom corners of

each board touch, the bottom of the slats will take on a natural

"curvature" as well.  This is desirable, and part of the

overall scheme and design of the chair.

Now, measure down 17-1/4" from the top center of the center

slat and make a mark on the slat.  The mark should be in the

CENTER of the board.  Using this point as the center of a

circle, draw a 17" radius circle across the top of the seven

slats, using either a beam compass of a pencil tied with

string.  These seven marks are the top of the slats and will

take on a uniform curve when installed according to the

directions that come later in this description.

<< see back1.bmp and front1.bmp for shape of upper and lower

chair back >>

Step FOUR              Cutting

After laying out all of the pieces for both the chair and

footstool (34 pieces for JAKE'S CHAIR and 21 for the

footstool) first cut out the irregularly shaped pieces with

the jig saw.  The remaining straight cuts can then be made

with the circular saw.  You will soon see that aligning the

most frequent pieces, such as seat slats and back slats

permits the longest and easiest rip cuts.  In addition, you

will soon begin to wish you had a table saw.  Perhaps your

spouse should be watching. . .

When laying out the arms of the chair <<p2.bmp>>, remember that

the left and right arms are mirror images of one another.  Since wood

will warp slightly when damp, it is necessary to orient the left and right

arms with the annular rings "up" (also known as bark side up) so

that swelling will cup the sides or the arm downward and center

upward, thus permitting water to run off more easily.

Holding the sanding board with a foot or knee, rub the ripped

pieces back and forth on the sandpaper (at an angle to the

grain) until the edges are straight.  Those of you with a long-bed

jointer will find this operation easier.  Ends should be sanded

as well, since the router's pilot bearing rests on the smooth

edge to guide the bit.  Curved edges can be smoothed by a

sanding block, but some people might prefer to use a drum

sander in the drill for this.  The sanding drum is

inexpensive and improves the outcome.  Thin saber saw blades

often bend sideways with use.  This leaves a cut that slopes.

Be very certain that the cut edges are square to the face of

the lumber, and not sloping.  Those of you with a belt sander

know that the rounded end of the sander will sand out curves

well.

Step FIVE           Routing

Place the routing pad on the workbench and round over the

edges ONLY IN THE AREAS SHOWN on the plans. NOTE:

ON THE  PLANS, ROUNDED AREAS ARE SHOWN WITH

A DASHED LINE AS OPPOSED TO A SOLID ONE.  Edges

that are to be joined with other pieces are left square,

for the most bearing surface and to improve looks.  If you

have a router table, all routing can be done on the table.

Always feed the work slowly against the rotation of the

router bit.  When working with the softer woods like

Cypress and Redwood, remember to go just fast enough

to prevent burning.

Step SIX             Sanding

Even the best router bit will occasionally leave a rough edge

on softwoods.  The only solution is to take a sanding block

with 100 grit paper and start to work.  If you have a belt

sander, I recommend clamping the sander upside down on your

workbench to make a miniature table sander.  100 grit belts

will quickly smooth out the roughest edges and will even give

a reasonable finish to the flat surfaces.  The round ends of

most belt sanders fit well into the curved areas.

***Suggestion- The easiest time to do your sanding is before

beginning the assembly of the chair or footstool.  While

there is the tendency to "see how it's going to look,"  the

hour or so taken in finish sanding all pieces will result in

a much more refined and comfortable chair.

Step Seven         Initial Drilling

Again, my assembly technique differs slightly from the

traditional method.  Since Jake's Chair allows freedom of

thickness in the selected material, the location of screw

pilot holes will also differ slightly to "hit" the middle of

the adjoining piece.  The location of the screw holes at the

end of the lower back support are a good example.  In my own shop,

I prefer to countersink (for screw heads) once the piece is ready

to be installed.  That way, I can consider which side of the wood

appears to go best on the surfaces.  My advice to new builders

is to mark the location of screw holes, but to wait until ready

for assembly before drilling or countersinking the holes.

The four "interior" screw holes in the lower back support,

all holes in the front legs, arm runner and arms may be

predrilled from the pattern.  However, the outer screw holes

on the lower back support and seat slats should be drilled to

"hit" the middle of the seat frame edge.  I prefer drilling

only as the pieces are clamped for assembly, except as noted

below.

At this stage, drill the holes with a 1/8" bit, and be sure

that scrap lumber is beneath the hole when the drill bit

exits the piece.  That way, the bit will not splinter the

wood as it cuts through.

The 1-1/2" seat slats have one hole at each end to join them

to the seat frame and the front (2") seat slat has two holes

at each end for more strength.  Seat slats and the slats in

the footstool warrant predrilling and countersinking, due to

the narrow wood's tendency to split.

Step Eight          Jake's Chair Assembly

Begin the assembly by placing the seat frames upright,

parallel and square 27" (outside measurement) from each

other.  Place the lower back support on the top of the seat

frame with the "ears" firmly against the stops.  The ends of

the lower back support should be flush with the sides of the

seat frames. << side1.bmp and back1.bmp >>

Using the approximate hole locations from the pattern of the

lower back support. drill through the lower back support and

into the seat frame with the 1/8" bit.  Then, remove the

lower back support and redrill the four holes in it using the

#6 or #8 countersink and pilot.  DO NOT USE THE PILOT ON THE

SEAT FRAME HOLES.  The screw heads should be just slightly

recessed.  Because of the direction the grain runs, do not

overtighten.

The reason for redrilling the pilot hole and countersink in

just the upper piece is that the new pilot hole will be too

large in diameter to permit the screws to firmly grip

the seat frame.  This is ideal in assembly, since tightening

the screws will permit thread slippage in the upper piece and

the smaller 1/8" hole in the lower piece will cause the upper

to be drawn tightly against the lower piece.

This technique is used throughout the entire assembly.

Glue the area of contact with exterior glue and then insert

the 2" screws in the holes and drive them home.

Before installing the 2" by 27" front seat slat << front1.bmp

and side1.bmp>>, place the front edge of the seat frame on

scraps of 1/4" material to raise it slightly above the

workbench.  Since the slats all have rounded edges, this will

permit the rounded portion of the first slat to slightly

"overlay" the front of the seat frame and to hide its lower

front corner.

Position the wide (2" by 27") seat slat against the front of

the seat frame <<front1.bmp >>.  Like the lower back support,

it should be flush with the sides of the seat frame with the

lower edge flat against the workbench, 1/4" lower than the

seat frame.  << side1.bmp >> Again, drill two 1/8" pilot

holes in each end and into the seat frame.  Remove the front

seat slat and redrill it with the countersink pilot bit.

Finally, attach it to the seat frame firmly with glue and

four 1-5/8" screws and then retighten the four 2" screws in

the lower back support.

The next step is to turn the seat over and install

the two "L"-shaped corner brackets under the lower back

support to hold it firmly at right angles to the seat frames.

<< bottom1.bmp and back1.bmp >> The screws run from the

lower back support down into the corner brackets and from the

seat frame into the corner brackets.  While it would be possible to

hide the screws more effectively by installing them from the

underside, the counterbore and countersink would reduce the

strength of these small pieces too much. Be  careful to drill

clearance  holes and countersink into the corner bracket,

or splitting will occur.

The corner braces should be adequate to force the two seat frames

into square, but be careful that the seat frames also remain

parallel with each other and are not racked into a parallelogram

shape..

Now, using two small scraps of 1/4" material as spacers

between each seat slat, begin at the front slat and

glue/screw each seat slat to the seat frame. The only wide

seat slat is the one used as the starter at the front.

All others are the narrow ones.  As you approach the back

support in the rear, adjust your spacing to give  an even

spacing between the final 2-3 slats, with any over/under

remaining on the seat frame being hidden under the lower

back support. This area is hidden from view in the finished chair.

The narrow seat slats are important to the comfort of a Jake's

Chair.  When sitting in the chair, the slats actually "give" a bit

at the points of maximum pressure, much as a spring might "give."

The results are a wooden chair that actually conforms a bit to

the shape and weight of the person sitting in it.

Install the arm braces to the OUTSIDE, top,middle of the front

legs, being sure that they are flush with the top to support

the arms and arm runners, which are installed on the INSIDE

of the front legs. << side1.bmp, back1.bmp,front1.bmp>>

The screws run from INSIDE the front legs into the arm

braces.

Cut two scraps of wood 10-1/2" long and use them to support

the front seat slat 10-1/2" above the workbench.  This is

the height of the front slat above the workbench.  Using 4

screws and glue, clamp and then fasten the front legs to the

outside of the seat frame 3" back from the lower front tip

of the SEAT FRAME (not front slat), being sure that the legs

are plumb <<side1.bmp>>. The screws pass from inside the seat

frame out into the front legs.

Note:  It should be emphasized that the front edge of the

front leg should intersect the bottom of the seat frame

assembly at a point 3" rearward of the front seat frame end.

Now, glue, clamp and screw the arm runners to the INSIDE of

the front legs, << side1.bmp back1.bmp and front1.bmp>>

making sure that they are flush at the top and front of the

front legs, and are perfectly level with the workbench.  The

screws pass again from INSIDE the arm runners into the front

legs.

An alternate assembly technique (rather than building in

place) is to assemble the front legs, arm runners and arm

braces before attaching them to the seat frame.  I have done

it both ways and there are advantages to both techniques.  As your

experience in building Jake's chair increases, you will probably

find it preferable to build the arm/leg assembly separately.

The next step is to glue and screw the UPPER back support

between the arm runners << back1.bmp >>in the position shown

on the plans.  However, only ONE screw per side is installed and no

glue is used initially.  This permits it to swivel a bit to

meet the seat back at a right angle when it is installed.

Using the position of the FRONT screw shown on the plans, drill

through the arm runner and install a temporary 1-5/8" screw into the

upper back support.  The support should be initially tipped at about

a 30 degree angle to the workbench to approximate the angle of the

finished seat back.  The lower edge of the upper back support should

be approximately just "inside" the lower rear corner of the arm

runner.  The actual placement is not critical.

The center seat back slat is now installed, being sure that

the bottom of the slat is FLUSH with the bottom of the lower

back support, that it is in the center of the lower back

support, and that it is plumb.  Two screws attach it at both

the upper and lower back braces.  Remember to swivel the

upper back support so that it is at right angles to the slope

of  the back slat.

Now, install a second screw (1-5/8" TEMPORARY screw) through

the arm runners into the upper back support. Later, these screws

will be removed and replaced with 3-1/2" screws for more strength.

Installing the longer screws at this point would promote splitting

of the wood.

At this point, the arms are fastened to the arm runners.

<< front1.bmp and side1.bmp >> Pilot holes are predrilled as

shown on the plans for alignment with the arm runners and arm

braces.  Be careful to keep the holes above the middle of the

boards and use glue as well.  Remember, also, that the arms must

be installed "bark side up" to help reduce cupping when wet.

The rest of the seat back slats are now installed

<<back1.bmp>>.  Work from the middle slat outward, keeping

the INNER BOTTOM CORNER OF THE NEW SLAT touching

the corner of the existing slat.  There is NO GAP at the bottom

and about a 3/8" gap at the top.  Since the slats fan outward

toward the top by about 3/8" each, each slat will intersect an inner

slat corner slightly "lower down" on the lower back support

in an arc.  This is why the holes are not predrilled, since

there are too many variables in where the holes would be.

Use two screws at both the upper and lower back support

contact points.

The outermost seat back slats are intended to clear the

inside of the arms by about 1/4".  << front1.bmp >>  I

recommend spacing the slats all at once and adjusting them

for uniform gaps at the top.  The final top of the chair

back should measure about 26" at its widest point.

The outermost seat back slats are fastened to the LOWER back brace

using the same 1-5/8" wood screws as the other slats.  However,

the slats are attached to the UPPER back brace by drilling completely

through the slats and brace with a 1/4" bit and inserting the four

10-24 or 1/4-20 pan head bolts.  Secure them with a flat washer

and nut.  These bolts will supply a "clamping pressure" on the grain

of the upper back brace, preventing splitting when subjected to

stress or rain.

After the four bolts have been installed, remove the temporary

1-5/8" screws connecting the arm runners to the upper back brace

and redrill the pilot holes to the full depth of the bit.  Then,

carefully insert the four 3-1/2" screws for the permanent

installation.

Finally, turn the finished chair over and nail on two of the nylon

glides to each of the front legs and one to the flattened portion

of the seat frame.

Congratulations, you now own a wonderful Jake's Chair!!!!!

STEP NINE              Footstool Assembly

To assemble the footstool, the first step is to install the

corner braces to the center of one end of each riser.  Be

sure that the edge of the corner brace is flush with the end

of the riser.  Screw through the riser into the brace. Then,

glue, clamp  and screw the upper and lower footstool ends to the

12-1/2" tall risers, screwing from inside out. << side1.bmp >>

The frames are installed on the "outside" of the risers.

Now, stand the risers up and plumb them with the corner braces

to the inside on the top.  Screw one of the narrow slats across

the center of the top or the upper frame, covering up the corner

braces with the slat.   Once screwed to the frames, using

one screw on each end, screw two additional screws up from  the

corner brace into the bottom of the center slat. << front 1.bmp >>

.        Now, screw each of the two wide slats to the lower frame on either

side of the riser. This will help to keep the two risers parallel

and plumb for the remainder of the assembly.  << bottom1.bmp >>

Finally, install the remaining slats to the upper frame and lower

frame, spacing them equally. Remember to equalize the distance

between the risers and the  angled corner of the lower frame.

This is done by eye. << side1.bmp >>

I have found equalizing the appearance of the slats to be easier

by doing the following:  First set the slats on the ends of

the upper fame, then the slats on the angles and finally

"average" the slats between the angled slat and the one

previously installed in the middle.  If the plans were

adhered to carefully, the spacing should work out to the 1/4"

recommended. <<side1.bmp>>

The final step in assembling the footstool is to turn the

finished footstool over and nail on the four remaining nylon

glides.  Remember to nail them to the LOWER FRAME and not to

the lower slat.

Congratulation!  You have now finished the footstool, also.

--------------------------------------------------------------

Many folks will want to stain Jake's Chair or paint it.

While I originally elected to let my cypress versions weather

naturally, the original Jake's Chairs on Jake's front porch

were painted with white enamel.  After several years of

my own chairs natural weathering, I tired of the dark silver

patina.  The  chairs were pressure washed and stained with

Cabot's cedar-colored deck stain. <<deck1.bmp>> The choice

is highly personal and about any solution is acceptable.

If you elect to paint Jake's Chair and footstool, first prime

the wood with an oil based wood primer, such as Rustoleum for

Wood.  Finish with an enamel of the desired color.  Should a

color other than pure white be used, clearcoating the chair

with a spar polyurethane can be done for added weather

resistance.  However, the spar polyurethane has a dark yellow

cast and will change the color of the chair appreciably.

Staining is best done with an oil based wiping stain or wood

dye.  After applying the stain to a small area with a sponge

brush and allowing it to soak in, wipe the area with a dry

rag to remove any excess. Follow up with two coats of clear

polyurethane for indoor use or the darker spar polyurethane

for outdoor use.  Finally, sand the cured finish (about 7

days) with 220 grit paper to remove any raised grain and then

apply one more coat.

Another process would be to use an outdoor penetrating deck

stain (actually a linseed oil and dye combination) that

contains a UV blocker.

SUMMARY

Since Jake's Chair plans are made available by FTP or E-Mail

over the Internet as an experiment in free dissemination of

plans electronically, I'd sure appreciate any comments or

refinements in what we've done. Feedback would benefit future

builders of this fine chair and footstool. Please feel free

to E-Mail your comments or pictures of your finished Jake's

Chair to me at:

scoundrl@mindspring.com

My home address is

Tom Gauldin

12333 Wingspread Way

Raleigh NC 27614-9245

I have a fax at (919) 676-1404.

A very special thanks goes out to Don Peterson

(donp@donp.boi.hp.com) for the tremendous work he did in

bringing Jake's Chair to life with his CAD program and for

all the time he spent measuring and debugging the patterns.

I want to also thank John O'Brien for his feedback on the construction

of an actual chair from the original plans Don and I prepared. As the

"beta" tester,  his frustrations and observations helped me to refine

the text of this project.  Thanks, fellows!

TAG

Thomas A. Gauldin        Here's to the land of the longleaf pine,

Raleigh, NC              The summerland where the sun doth shine,

scoundrl@mindspring.com  Where the weak grow strong and the strong grow great,

FAX (919) 676-1404       Here's to Downhome, the Old North State.

Footnote:

CHANGES to JAKE'S CHAIR- the following has now been incorporated in

the text of the article.  The footnote is for folks who may have

already built Jake's Chair and might want to make the changes without

having to re-read the entire article.

-----------------------

When I was first designing the Jake's Chair and footstool, I built

chairs from scrap wood to test out the fit and feel of the chair.

It took about six "generations" of trial and error chairs until I

felt that I had the best combination of comfort and ease of

construction.  Some of the original Jake's Chairs are now almost

ten years old, and I recently had the great opportunity to examine

a couple that had been continuously exposed to the elements for

those ten years without any benefit of maintenance.

The chairs in question had been in use during all that time, had

been left to remain outside during winter months, and had been

subjected to some harsh use by college students, including one who

weighed in at over 350 pounds. Both chairs were still functional,

though one had to have its back propped up by a board to prevent

it from becoming a recliner.  By examining the chairs, I made a

couple of observations where the original design could be

improved.

I want to share those improvements with you, and will also be

including this note in the file jake.txt, which is included in the

master file, jake.zip, in my FTP site.

If you have built a Jake's Chair, you might consider making these

modifications NOW, instead of making them as a "fix," later on.

Both modifications center around what I call the "UPPER BACK

SUPPORT" in the plans and text.  Since the upper back support is

rounded and cut from a wide board, it can split along the grain

line when subjected to long-term bad weather or abuse.  Folks who

have built the chair may recall that the seat back slats are

fastened to the lower and upper back support with 1-1/2" screws.

The solution to the splitting  is to merely replace the two short

screws holding the outermost seat back slats to the upper back

support, with small bolts that run THROUGH the entire support,

with a washer and nut on the rear.  The bolts will subject the

wood to sufficient clamping pressure that it should not split with

age.  In a pinch, even a 3" wood screw would probably be

sufficient to prevent splitting.  I recommend either using either

1/4-20 or 10-32 pan head screws or carriage bolts.  4" lengths

would be more than adequate, with the excess being cut off and

smoothed up with a file.

The second problem I observed, again involved the upper back

support, but with splitting out where it was fastened to the arm

rest supports.  There, my original design used a 1-1/2"  and a 2"

wood screw driven into the end of the support from the arm rest

support.  While there may be more elegant solutions, my remedial

action was to merely reposition the pieces slightly, drill new

pilot holes, and substitute in 3-1/2" screws.  These pulled down

really well and should have been made a part of the original

design.  The only caution is that the pilot hole for the screws

should NOT intersect the outermost carriage bolt that is holding

the seat back slat.  The screws should be installed AFTER the seat

slat bolts have been installed, to prevent splitting.

While not as important as the changes noted above, I have also found

that the use of nail-on nylon button glides to the legs of the chair

and base of the footstool are a good touch.  The button glides hold

the wooden base of the chair up a tad, permitting it to dry faster

after a rain, and the gap also prevents wicking of water into the

exposed ends of the legs.  When stained, the nylon buttons also prevent

the chair's color from rubbing off onto the deck or patio.

On a more pleasant note, my dear wife pointed out a feature of the

Jake's Chair that I had always taken for granted, but had not

mentioned in print.  That feature is that the Jake's Chairs can be

stacked!  During winter months, some folks may want to retire

their Jake's Chairs to the basement or to a shed.  Since the arms

are so wide and deep on the Jake's Chair, one chair may be placed

on top of another to reduce the amount of floor space required.

I've been doing it for years, though I never thought to mention it

to others.

I sincerely apologize to the thousands of folks who have written

me to tell me about having built a Jake's Chair for not having

caught the design problem with the upper back support earlier.

However, the "fix" is so simple and quick that I doubt if anyone

will object to making the improvements.

Tom Gauldin, Raleigh NC

scoundrl@mindspring.com